City of Barstow Erosion Control Notes:

- THIS EROSION CONTROL PLAN AND THE STORM WATER POLLUTION PREVENTION PLAN (SWPPP, IF APPLICABLE) ARE PREPARED FOR THIS PROJECT ARE BASED ON THE NATIONAL POLLUTANT DISCHARGE ELIMINATION SYSTEM (NPDES) GENERAL PERMIT FOR WASTE DISCHARGE REQUIREMENTS FOR STORM WATER DISCHARGES ASSOCIATED WITH CONSTRUCTION AND LAND DISTURBANCE ACTIVITIES, ORDER NO. CAS000002, STATE WATER RESOURCES CONTROL BOARD CALIFORNIA CONSTRUCTION GENERAL PERMIT ORDER NO.: 2022-0057-DWQ, AND THE PHASE II SMALL MUNICIPAL SEPARATE STORM SEWER SYSTEMS (MS4s) ORDER NO.: 2013-0001-DWQ.  INFORMATION IS FOUND AT: https://www.waterboards.ca.gov/water_issues/programs/stormwater/construction/general_permit_reissuance.html, And https://www.waterboards.ca.gov/water_issues/programs/stormwater/phase_ii_municipal.html

- ALL SWPPP REQUIREMENTS, SPECIFICATIONS, AND BMP'S SHALL BE IN ACCORDANCE WITH THE LATEST EDITION OF THE CALIFORNIA STORMWATER BMP HANDBOOK FOR CONSTRUCTION GUIDELINES AND AMENDMENTS THEREOF.   BEST MANAGEMENT PRACTICES ARE FOUND AT: https://www.casqa.org/, IN ADDITION TO CITY OF BARSTOW STANDARDS AND REGULATIONS AS LONG AS THOSE STANDARDS CONFORM WITH THE REQUIRED ELEMENTS OF THE STATE WATER RESOURCES CONTROL BOARD (SWRCB).

- PEDESTRIAN AND TRAFFIC SAFETY SHALL BE IN ACCORDANCE WITH STATE STANDARDS AND REGULATIONS AND SHALL BE ESTABLISHED PRIOR TO THE START OF CONSTRUCTION.  IT IS THE LEGAL RESPONSIBLE PERSON AND DEVELOPER’S RESPONSIBILITY TO ADDRESS ANY BMP ISSUES THAT MAY JEOPARDIZE THE SAFETY OF SUCH AND BRING TO THE ATTENTION OF THE SIGNING ENGINEER AND CITY ENGINEER PRIOR TO INSTALLATION.

- THIS PLAN AND SWPPP SHALL BE POSTED IN CONTRACTORS TRAILER/OFFICE OR WHERE ALL CONTRACTORS AND SUBCONTRACTORS MAY HAVE EASY ACCESS AND BECOME FAMILIAR WITH ITS CONTENTS PRIOR TO BEGINNING WORK.

- THE DEVELOPER IS RESPONSIBLE FOR FOLLOWING THE CITY’S MUNICIPAL SEPARATE STORM SEWER SYSTEMS (MS4) PERMIT AND MAINTAINED STORM DRAINS AND CHANNELS THAT MAY BE AFFECTED BY THE PROJECT. THE USE OF STRAW FIBER ROLLS OR SANDBAGS SHALL BE CAREFULLY MONITORED FOR DAMAGES AND DISPOSED OF IN SUCH A MANNER TO PREVENT BMP MATERIALS, DEBRIS OR SAND FROM ENTERING THE DRAINAGE SYSTEM.  STORM DRAIN PROTECTION SHALL ENSURE SEDIMENTS STAY OUTSIDE MS4 DRAIN INLETS, BUT ALLOW STORM AND NON-STORM WATER FLOWS IN.  STORM DRAIN INLETS AND CONVEYANCES SHALL BE INSPECTED PRIOR TO RAIN EVENT AND ENSURE THE STORM DRAIN IS NOT OBSTRUCTED WITH DEBRIS, SEDIMENTS, OR BMP MATERIALS, THUS FLOODING OR CAUSING UNDUE DAMAGES.

- CONSTRUCTION ACTIVITIES IN THE RIGHT OF WAYS, MS4, AND FLOOD CONVEYANCES SHALL BE CONDUCTED DURING DRY WEATHER ONLY.  BMPS PLACED IN THE RIGHT OF WAYS, AND MS4 SHALL NEVER BE LEFT UNATTENDED.  DO NOT BLOCK OR IMPEDE FLOODWAYS AND CATCH BASINS WITHOUT WRITTEN PERMISSION FROM CITY ENGINEER, OR FLOOD CONTROL JURISDICTION.

- A STAND BY CREW, AND ALL NECESSARY EQUIPMENT, SHALL BE AVAILABLE AT ALL TIMES DURING THE RAINY SEASONS.  WHENEVER THE LIKELIHOOD OF RAIN IS IMMINENT, NECESSARY MATERIALS SHALL BE AVAILABLE ON SITE AND STOCKPILED AT CONVENIENT LOCATIONS TO FACILITATE THE RAPID CONSTRUCTION OF EMERGENCY CONTROLS.

- ALL AREAS SHALL BE MAINTAINED IN SUCH A STATE THAT FIRE DEPARTMENT ACCESS IS AVAILABLE AT ALL TIMES, INCLUDING ACCESS TO NEIGHBORING PROPERTY.

- Site inspections and appropriate maintenance of all erosion control measures/devices shall be conducted and documented at all times during construction (especially prior to, during, and after rain events).  BMP INSPECTIONS SHALL BE MADE AVAILABLE AND SUBMITTED TO THE CITY SWPPP MONITOR UPON REQUEST.

- The developer/LRP shall be responsible for the placement and maintenance of all erosion control measures/devices as specified by the approved plan until such time that the project is accepted as “complete” by the CITY OF BARSTOW ENGINEERING DEPARTMENT or until released from the Conditions of Approval of their General Permit(S).

- Erosion control measures/devices may be relocated, deleted or additional measures/devices may be required depending on the actual conditions encountered during construction. Additional erosion control measures/devices MAY BE REQUIRED at the discretion of the CITY Engineer, CITY SWPPP Monitor, or RWQCB Inspector.  THIS INCLUDES, BUT NOT LIMITED TO, ADDITIONAL STREET/PEDISTRIAN PATH SWEEPING, ADDITION SEDIMENT CONTROLS OR UPGRADES, AND OTHER GOOD HOUSEKEEPING MEASURES NECESSARY.

- In the event of a failure, the developer and/or his representative shall be responsible for cleanup and all associated costs or damage. In the event that damage occurs within the CITY right-of-way, OR PRIVATE PROPERTY, AND the CITY OF BARSTOW is required to perform cleanup, the owner shall be responsible for CITY reimbursement of all associated costs or damage(S).

- In the event of failure and/or lack of performance by the owner, DEVELOPER and/or contractor to correct erosion control related problems the CITY ENGINEERING Department may revoke all active permits and recommend that CITY Code Enforcement provide a written notice or stop work order.

- dust/wind control measures shall be used throughout the duration and in all areas of disturbance, including unpaved paths of travel and staging areas.  install 6-FOOT-HIGH screening fabric fencing along perimeters of public right of way/pedestrian paths.

- UPON IMPLEMENTATION AND INSTALLATION OF ANY BMPS, THE CONTRACTOR AND SUBCONTRACTORS SHALL ADDRESS ANY CONCERNS OR QUESTIONS REGARDING THIS PLAN AND THE SWPPP PROVIDED FOR THIS PROJECT WITH THE LEGAL RESPONSIBLE PERSON (LRP) AND THE SIGNING CIVIL ENGINEER, THIS INCLUDES, BUT IS NOT LIMITED TO, LOCATION OF BMPS, INSTALLATION REQUIREMENTS, EQUIPMENT NEEDS, AND MATERIALS TO BE USED.  ANY CHANGES TO THOSE SPECIFIED SHALL BE BROUGHT TO THE ATTENTION OF THE QSD AND CONTRACTOR SHALL ASSUME FULL RESPONSIBILITY.

- The developer shall provide the CITY OF BARSTOW with the Waste Discharge Identification Number (WDID #) or with verification that an exemption has been granted by RWQCB. WDID No.: ____________________________.